Introduction to Computer Vision

Coursework

Submission 2

Your name 		________ HAO BAI ________

Student number	 ________201218765__________

Question 4(a)

It+1

It

Motion field of It+1

Question 4(b)

Pt+1

It+1

Your comments

How to use this function:

Run cell 4-a and cell 4-b in ICV_Assignment2_main_file.m file.

The algorithm in this cell is that:

- The function will load the video dataset(DatasetC.mpg) and find the 9th and 10th frame as frame It and It+1.

- The block size is 16*16 the search windows size is 20*20.

- Using the ICV_blockMatch () to estimatie the motions, this function required enter a the It and It+1 images and block size and search windows size.

- To predicte the It+1 images, the ICV_predictionOfFrame () can predicte the It+1 images as Pt+1. The function required enter a the It and It+1 images and block size and search windows size.

The algorithm of the using functions in these two cells:

ICV_blockMatch ()

- Convert the images matrix from uint8 to double.

- To divide the whole image into several equally sized non-overlapping blocks, the function will cut the extra part of the image.

- The function cut the image into equally sized non-overlapping block according to the block size. The loop will begin with the first pixel of the image end with the last pixel of the image, the step size is the block size.

- Each block matrix size is equal to img(i:i+block_size-1, j:j+block_size-1)

- Store the block in img_block(block_i, block_j).matchBlock , the block_i and block_j is the count that the number of the blocks. And store the image location in the img_block(block_i, block_j). loc.

- Find the size of the searching windows. The block is in the center of the searching windows, when the beginning of the block is 1 or the ending is the last number of the image, the search windows will out of bounds.

- To fix that, the function will check the location of the block. if block is on the boundary of the image, the searching windows size will smaller size to fit the block window (delete the out of bound part).

- The reference of the search windows is the It+1 image.

- The searching window moving the block pixel by pixel, and calculate each blocks’ mean square error.

- And then find the minimum mean square error in the searching window, and it’s location.

- Using the minimum mean square error location to calculate the pointer location.

- Then after all block has been calculate, the function will be using quiver function to draw the motion estimate pointer. Using the pointer location and the block size. And show in image.

ICV_predictionOfFrame

- This function when doing the block matching and motion estimate is same as the ICV_blockMatch function.

- To predicte the next frame, the block will move to the estimated location.

- If the location is out of bound, the function will cut the extra block.

- If the pointer location is not equals to 0, the image block will over-write the pointer location block.

Question 4(c)

Pt+1

Block size = 8x8

Pt+1

Block size = 16x16

Pt+1

Block size = 4x4

Your comments:

How to use this function:

Run cell 4 - c :  16x16 search window, block_Size 4x4 and 4 - c :  16x16 search window, block_Size 88 and 4 - d :  88 search window, block_Size 88 in ICV_Assignment2_main_file.m file.

According to the result:

block 4*4:

block 8*8:

block 16*16:

The smaller block size will cause accurate moving object location calculate. But the object will be breaked into pieces. That because more pointer will be included in the motion estimating. The block size that same as the search windows size will cause can not making the motion estimating. So the predition frame will same as the previous frame.

Question 4(d)

Pt+1

Window size = 16x16

Pt+1

Window size = 32x32

Pt+1

Window size = 8x8

Your comments:

Windows size 8*8

Windows size 16*16

Windows size 32*32

The block size that same as the search windows size will cause can not making the motion estimating. So the predition frame will same as the previous frame. The biger size searching windows will generate accurate result than the smaller sized searching windows. More accurate pointer generated.

Question 4(e)

Plot graphs:

Time versus window size

Time versus block size

Your comments:

Block size	Search windows size	Time

4*4		16*16			1.761944s

8*8		16*16			0.824722s

16*16		16*16			0.679418s

Block size	Search windows size	Time

8*8		8*8			0.705818s

8*8		16*16			0.759334s

8*8		32*32			1.690244s

As the graph shows that, the smaller block size the more time needed. The biger search windows size the more time needed. That because the smaller size block windows and biger size search window need more loop to calculate the pointers, and amount of the pointers are more.

Question 5(a)

Original frames:

Selected frame 2

Selected frame 1

Reference frame

Frame differencing:

Threshold results: Threshold = 40

Your comments:

How to use this function:

Run cell 5-a and cell 5-b in ICV_Assignment2_main_file.m file.

The algorithm in this cell is that:

To calculate the frame differencing and apply the classification, function ICV_captureMovingObjectFirstFrame taken to be used. This function required a video path and the threshold.

The algorithm of the using functions:

ICV_captureMovingObjectFirstFrame

- Loading the video and get the amount of the frames

- Looping from the first frame to the last-1 frame.

- Compare each pixel of the two frames, in this function compare the first frame and other frames.

- If in same location, two frames are different. And the different number is less than the classification threshold number. This location of pixel is the moving object. And set the color of the pixel equals to white (255). If the location of the two frame is same or less than the classification threshold the color of the pixel is set to black (0).

- Show the calculated images.

Observed

The higher classification threshold, the smaller and gatherd the moving objects.  Both of the 1st frame cars and the ith frame cars will shown on the resulting image.

Question 5(b)

Original frame:

Selected frame 1

Selected frame 2

Frame differencing:

Threshold results:

Your comments for 5a,5b:

How to use this function:

Run cell 5-a and cell 5-b in ICV_Assignment2_main_file.m file.

The algorithm in this cell is that:

To calculate the frame differencing and apply the classification, function ICV_captureMovingObjectPreviousFrame taken to be used. This function required a video path and the threshold.

The algorithm of the using functions:

ICV_captureMovingObjectPreviousFrame

- Loading the video and get the amount of the frames

- Looping from the first frame to the last-1 frame.

- Compare each pixel of the two frames, in this function compare the i frame and i+1 frames.

- If in same location, two frames are different. And the different number is less than the classification threshold number. This location of pixel is the moving object. And set the color of the pixel equals to white (255). If the location of the two frame is same or less than the classification threshold the color of the pixel is set to black (0).

Observed

Both frame cars will shown on the resulting image, the resulting image cars are are both the I frame car and I+1 frame car.

Question 5(c)

Generated background

Your comments:

How to use this function:

Run cell 5-c in ICV_Assignment2_main_file.m file.

The algorithm in this cell is that:

Using the function ICV_generateBackground(video_path) to calculate the background image. This function required a video path.

The algorithm of the using functions:

Question 5(d)

Bar plot

Your comments:

Question 6(a)

Three non-consecutive windows

W3

W2

W1

LBP of windows

LBP3

LBP2

LBP1

Histograms of LBPs

H3

H2

H1

How to show the result:

Run the cell 6- a in ICV_Assignment2_main_file.m file.

The algorithm in this cell is that:

- Load the face image(face-2.jpg) and convert to grayscale mode using rgbgray function

- Set the Block size = 128, that means each block windows is 128 pixels * 128 pixels. That divided the face image into 4 small windows.

- Using ICV_divideIntoNonOverlapping functions. That required input an image and the block size. And return a group of non-overlapping images that divided from the images.

- Using size () function to get how many blocks, the image has been divided.

- The loop is to read each blocks and handle.

- The ICV_LBPfunction is a function that required input a image and return a LBP image.

- Then function ICV_hisgram will print the histogram of the frequency of the numbers, the required input is the block images and number of the all block images and the current count number. That will help the function draw the bar chart together.

- Each of the histogram will as the feature descriptor of each block.

The algorithm of the using functions:

ICV_divideIntoNonOverlapping:

- This function will divide the big image into small piece of the non-overlapping images as the return value accordong to the block size needed.

- The function will get to know the length and width of the image

- sometimes the image cannot cut into equally sized non-overlapping block as the asked size, the function will cut extra length or width.

- The function will divide the image into small block, the step size is the block size. Each block will have stored in the return value.

- The block size is 128 pixels, so the return value will be a 128*128*4 matrix.

ICV_LBPfunction

- This function compares 8 of each pixel neighbors with the pixel self. If the number larger or equals to the pixel, then the neighbors number equals to 1. Else, the neighbors number will equals to 0.

- And then each pixels equals to the numbers that converted counter-clockwise neighbors binary to decimal number.

ICV_LBPfunction

- The function will count the of the gray level and the total number of the pixels.

- The function will calculate the frequency of each numbers and using bar chart to show.

- Normalize the histogram and show in the same sub-graph.

Question 6(b)

Two example images:

Car image

Face image

Car descriptor

Face descriptor

Descriptors:

Your comments:

How to show the result:

Run the cell 6- b in ICV_Assignment2_main_file.m file.

The algorithm in this cell is that

- Load the face and car image (face-2.jpg and car-1.jpg) and convert to grayscale mode using rgbgray function

- Set the Block size = 128, that means each block windows is 128 pixels * 128 pixels. That divided the face image into 4 small windows.

- Using ICV_divideIntoNonOverlapping functions. That required input an image and the block size. And return a group of non-overlapping images that divided from the images.

- Using size () function to get how many blocks, the image has been divided.

- The loop is to read each blocks and handle.

- The ICV_LBPfunction is a function that required input a image and return a LBP image.

- Then function ICV_hisgram will print the histogram of the frequency of the numbers, the required input is the block images and number of the all block images and the current count number. That will help the function draw the bar chart together.

- Each of the histogram will as the feature descriptor of each block.

- Combine each of the sub-histogram in to one histogram that is the descriptor of the whole image.

- To combine the frequency of each of the number occur, the function will add all of the frequency first and divide by the number of the blocks.

The algorithm of the using functions (same as the 6 -a):

ICV_divideIntoNonOverlapping:

- This function will divide the big image into small piece of the non-overlapping images as the return value accordong to the block size needed.

- The function will get to know the length and width of the image

- sometimes the image cannot cut into equally sized non-overlapping block as the asked size, the function will cut extra length or width.

- The function will divide the image into small block, the step size is the block size. Each block will have stored in the return value.

- The block size is 128 pixels, so the return value will be a 128*128*4 matrix.

ICV_LBPfunction

- This function compares 8 of each pixel neighbors with the pixel self. If the number larger or equals to the pixel, then the neighbors number equals to 1. Else, the neighbors number will equals to 0.

- And then each pixel equals to the numbers that converted counter-clockwise neighbors binary to decimal number.

ICV_LBPfunction

- The function will count the of the gray level and the total number of the pixels.

- The function will calculate the frequency of each numbers and using bar chart to show.

- Normalize the histogram and show in the same sub-graph.

Question 6(c)

Block diagram of classification process

Face image each block descriptor and the whole image descriptor intersection value.

Car image each block descriptor and the face whole image descriptor intersection value.

Your comments:

To classify each of the block, the script will calculate the intersction of the two histograms (the his tograms generated from the pervious cell 6- b.) of the car image and the face image (face-2.jpg and car-1.jpg) first. The intersection value is the sum of each bars value (The two histograms of each image is the probbility.), the two-image intersection value is 0.6854.

The intersection value is calculate by:

Then calculate two different image the block histograms and the whole face histogram intersection value. According to the graph shows that:

- Face image each block descriptor and the whole image descriptor intersection value.

- Car image each block descriptor and the face whole image descriptor intersection value.

The graph shows that the same image different block intersection value is higher than 0.8. If using the different image blocks, the intersection value will be lower than 0.7. So that the block classify can be done by calculating the intersection value of each block and the whole image.

Question 6(d)

Your comments:

When decreasing the windows size, the size decreases to 64 pixels * 64 pixels block.

The two-image (face-2.jpg and car-1.jpg) intersection value is not changed (0.6854). How ever the intersection value of the small block (64 pixels * 64 pixels block) decrease:

- The intersection value of the face image histogram and the first block of the face image is: 0.6789

- The intersection value of the face image histogram and the first block of the car image is: 0.4843

That shows that the small windows size will only decrease the intersection value of the image and the block histogram. The number of the intersection values can be used to classify the images.

Question 6(e)

Your comments:

When increase the windows size, the size increase to 200 pixels * 200 pixels block.

- The intersection value of the face block and face image increase is 1.0

- The intersection value of the car block and face image increase is 0.6897

When the block size is 128*128 pixels the intersection value of the face block and face image is larger than 0.8, and the intersection value of the car block and the face image is always lower than 0.7. It shows that as the block size increase the intersection value of the image and blocks increases and the accuracy of the classify increase.

Question 6(f)

Your comments